01

NAME : KAVIYA.R

REG NO : 192521075

Application Based

AA

CSA1221- COMPUTER ARCHITECTURE

APPLICATION BASED

Saru Hasini

1. Smart City Surveillance System

Answer:

A smart city surveillance system continuously transfers video data between I/O devices (cameras), memory (RAM), and the CPU. During peak hours, multiple cameras generate large amounts of data simultaneously, causing congestion.

Interaction between CPU, Memory, and I/O

- I/O Devices: Cameras capture and send video streams.

- Memory (RAM): Temporarily stores incoming video frames.

- CPU: Fetches data from memory, processes it, and performs threat detection.

If the CPU cannot process data as fast as it arrives, memory becomes overloaded, leading to delays.

Single-Bus vs Multi-Bus Structure

Single-Bus System

- CPU, memory, and I/O share one common bus.

- Only one data transfer occurs at a time.

- Causes bus contention and slower performance.

Multi-Bus System

- Separate buses are used for CPU-memory and I/O communication.

- Multiple transfers occur simultaneously.

- Reduces waiting time and improves throughput.

Improving the Instruction Execution Cycle

- Use instruction pipelining to overlap fetch, decode, and execute stages.

- Increase cache memory to reduce RAM access time.

- Use DMA (Direct Memory Access) for transferring video data directly to memory.

- Employ multi-core processors to process multiple video streams simultaneously.

Conclusion

A multi-bus architecture, larger cache, DMA, and pipelining significantly reduce processing delays and improve real-time surveillance performance.

2. Real-Time Stock Trading Application

Answer:

The stock trading application executes millions of instructions every second. High CPI (Cycles Per Instruction) increases transaction delays.

Impact of the Fetch–Decode–Execute Cycle

- Fetch

- CPU retrieves instruction from memory.

- Cache misses increase fetch time.

- Decode

- CPU interprets the instruction.

- Complex instructions increase decoding time.

- Execute

- Arithmetic, memory access, or branch operations are performed.

- Branch instructions may flush the pipeline, increasing CPI.

Reasons for High CPI

- Frequent memory access

- Cache misses

- Branch instructions

- Pipeline stalls

Methods to Reduce CPI

- Increase cache size.

- Use branch prediction.

- Apply instruction pipelining.

- Use superscalar processors.

- Optimize software to reduce unnecessary memory access.

- Improve compiler optimization.

Conclusion

Reducing memory delays and branch penalties lowers CPI, increases CPU performance, and enables faster transaction processing.

3. 8085 Microprocessor – Industrial Temperature Control

Answer:

The 8085 microprocessor collects sensor readings and controls actuators. Incorrect addressing modes may result in wrong temperature values being processed.

Addressing Modes and Their Influence

Immediate Addressing

- Data is directly included in the instruction.

- Example:

MVI A,25H

Register Addressing

- Data is stored in CPU registers.

- Faster execution.

Direct Addressing

- Memory address is explicitly specified.

- Example:

LDA 2500H

Indirect Addressing

- Memory address is stored in a register pair.

- Example:

MOV A,M

Possible Causes of Errors

- Incorrect memory address

- Wrong register selection

- Uninitialized registers

- Sensor data stored in incorrect locations

- Programming mistakes

Using the 8085 Instruction Set Effectively

- Use correct addressing modes.

- Validate sensor readings.

- Initialize registers before use.

- Use comparison instructions (CMP).

- Handle abnormal readings using conditional jumps.

- Test the program thoroughly.

Conclusion

Proper addressing modes and correct instruction usage ensure accurate temperature monitoring and reliable industrial control.

4. 8086 Segmentation

Answer:

The 8086 microprocessor uses segmented memory to access up to 1 MB of memory.

Memory Segments

Code Segment (CS)

- Stores program instructions.

Data Segment (DS)

- Stores variables and data.

Stack Segment (SS)

- Stores function calls, return addresses, and local variables.

Extra Segment (ES)

- Used for additional data storage.

Problems Due to Improper Segment Handling

- Wrong data accessed.

- Stack overflow.

- Incorrect return addresses.

- Program crashes.

- Memory corruption.

Instruction Execution

The CPU calculates the physical address using:

Physical Address = Segment × 16 + Offset

Example:

DS = 2000H

Offset = 1200H

Physical Address = 21200H

Addressing Modes

- Immediate

- Register

- Direct

- Based

- Indexed

- Based Indexed

Correct use prevents memory access errors.

Ensuring Correct Execution

- Initialize segment registers correctly.

- Prevent stack overflow.

- Use PUSH and POP properly.

- Validate memory addresses.

- Use proper addressing modes.

Conclusion

Correct segment management ensures reliable execution and prevents memory-related errors.

5. Number System Conversion in Data Communication

Answer:

Packet data is commonly represented in hexadecimal because it is easier to read than binary.

Importance of Hexadecimal and Binary Conversion

- Simplifies debugging.

- Makes packet analysis easier.

- Reduces representation length.

- Helps programmers identify instruction codes quickly.

Example

Hexadecimal:

3A

Binary:

00111010

Effects of Conversion Errors

- Wrong instruction execution.

- Incorrect packet interpretation.

- Data corruption.

- Communication failures.

- Security vulnerabilities.

Role of Efficient CPU Design

- Faster instruction execution.

- Improved cache performance.

- Better error handling.

- Higher throughput.

Role of Benchmarking

Benchmarking measures:

- CPU execution time

- Memory performance

- Cache hit ratio

- Network throughput

- System latency

It helps identify bottlenecks and optimize system performance.

Conclusion

Accurate hexadecimal-to-binary conversion, efficient CPU architecture, and regular benchmarking improve system reliability and minimize processing errors.

Summary

| Question | Key Answer |
| --- | --- |
| Q1 | Multi-bus, DMA, cache, pipelining improve surveillance performance. |
| Q2 | Reduce CPI using cache, pipelining, branch prediction, and compiler optimization. |
| Q3 | Proper 8085 addressing modes prevent incorrect sensor data processing. |
| Q4 | Correct 8086 segment management avoids memory errors and stack overflow. |
| Q5 | Accurate hex–binary conversion, efficient CPU design, and benchmarking ensure reliable data communication. |

These answers are suitable for assignment submission, with enough detail to earn full marks while remaining clear and concise.

Top of Form

Bottom of Form